REVERSING MEDIA ON SINGLE HEAD FLEXIBLE DISK DRIVES

typed by Keith Petersen, W8SDZ,

from information supplied by Dysan Corporation

There  has  been  a tendency by some end users  to  economize  by

attempting  to use the media on both sides in a single head  disk

drive.   We must not lose sight of the fact that the value of the

data  stored on diskettes exceeds the cost of the media by a wide

margin.   Loss of data on either read or write means time delays,

reconstruction  of lost data,  and customer dissatisfaction  with

the system,  drive and/or media manufacturer.  All of this can be

avoided in advance if the end user is made aware of the whys  and

why nots.

HEAD SHOE AND PAD OPERATION

The  relationship of the head to the media is such that when  the

jacket  is properly inserted,  and all interlocks are  satisfied,

the  head is loaded onto the media on the recording side,  and  a

felt loading pad is applied to the non-recorded side.   In normal

operation,  a  gradual  build-up of oxide will accumulate on  the

pressure pad.   There might even be some wear on the non-recorded

side due to a scouring action of the oxide impregnated pad.

If the media is reversed,  the scouring action will now occur  on

the  prime recorded side,  and the previously scoured side is now

presented  to an abrasive wearing by the contaminated  load  pad.

Since  this  data is not being read,  there is not any  means  of

detecting  the  amount  of wear or the loss  of  data.   While  a

catastrophic  failure might not occur,  it is possible that  some

drop-out or other read error might go undetected.   Worse yet, is

the  possibility that the error condition might be  intermittent,

which makes the entire operating system suspect.  Another adverse

effect  of  reversing  the  media,  is caused  by  reversing  the

direction  of  rotation of the media against  the  pressure  pad.

This  reversal of direction is apt to "break off" any build-up of

oxide  particles.   This presents a potential  loose  contaminent

situation.

The  net  effect  of this reversing (or flipping) action  over  a

period  of  time  is  to  reduce  performance  and  increase  the

probability of drop outs and errors.

DISKETTE TENSIONING

On  most  Floppy  Disk Drives,  when  the  diskette  is  properly

inserted  and  operation has begun,  pressure is applied  to  the

jacket  on  both sides so that proper tension is created  on  the

flexible media prior to the recording head.  This also provides a

wiping  action of the liner material against the flexible  media.

When  the  jacket  is reversed (or  flipped),  the  direction  of

rotation  is  reversed,  breaking loose any extraneous  particles

built-up by prior wiping.   Thus,  reversing the media  increases

the  probability of extraneous contamination and again  increases

the possibility of errors.

TWO HEAD DRIVES

The  above problem areas do not occur on two head drives that are

designed for two sided applications.   On a two head  drive,  the

pressure  pad  has  been replaced by a second head mounted  in  a

ceramic  shoe.   The operation now consists of a  head-media-head

relationship.  The soft pressure pad with possible oxide build-up

has been eliminated.

The diskette tensioning apparatus is the same on one and two head

drives.   Since media spin direction is not reversed by flipping,

the oxide break-off problem does not occur.

SUMMARY

The  foregoing summarizes the reasoning why Dysan and  major  OEM

suppliers of diskette drives do not recommend two sided media for

one  head  drive  application.   Dysan feels that  the  potential

operating  problems would make an unwarranted reflection  on  our

reputation  by  using media in an unsuitable fashion.   When  IBM

introduced  the  3740 diskette,  they  intentionally  interlocked

reversal  possibilities  by offsetting the index  hole  from  the

centerline.  IBM does not make a reversable diskette.

Dysan  does test and supply two sided media for operation in  two

head (two sides) disk drives.

############################################################################