#JUST ON

#offs 10

#SPACE 2

#TITLE INTERDOS DOCUMENTATION

<INTERDOS

[INTERDOS is a CP/M program for moving data files from North Star

disks to CP/M disks and back.  CP/M is a trademark of Digital Research,

INTERDOS is not a Digital Research product.  INTERDOS works by loading

the North Star DOS into memory while CP/M is loaded and making calls

between the two operating systems to do the disk I/O.  In addition to

simple file copy, INTERDOS will make certain format changes also.]

[The standard CP/M record format for ED or ASM files is the text line

followed by a carriage return and a line feed.  There are several formats

for North Star, but the one addressed in INTERDOS is the format used by

SECRTARY, XEK, and several old assemblers.  This format is character

count, four ASCII digits for the line number, and a carriage return.

INTERDOS will copy either from North Star to CP/M or CP/M to North Star.

If you are copying from North Star to CP/M, five conversion options are

available.  First, of course, is file-to-file copy without conversion.

Second, SECRTARY format to CP/M ASM format.  With this conversion the

line numbers are stripped off, tabs are inserted in certain locations

and both a carriage return and a line feed are added to the destination

line.  The third format is similar to the second, but no tabs are

inserted.  The fourth format is to create a file on CP/M that can be

read for CP/M BASIC.  This is called the SECRTARY to CP/M BASIC conversion

and is similar to the third option except that the line numbers are left

on.  The output record will contain the four ASCII character line number,

the text line without inserting tabs, and a carriage return and line feed

at the end of the line.  The last option is the North Star BASIC to CP/M

BASIC format.  North Star BASIC files contain the line number as a binary

two byte field, and the BASIC keywords are kept as one byte tokens.  This

conversion option converts the line number to four ASCII characters,

converts the tokens to the ASCII equivalent and adds a carriage return

and line feed at the end of the line.  NOTE: THE LINE NUMBERS IN THE BASIC

PROGRAM MUST NOT BE GREATER THAN 9999.  Renumber the lines in BASIC before

doing the file copy if necessary.  Also this only converts the lines and

not the actual code.  There are several differences between North Star

BASIC and CP/M BASIC which have to be changed before a North Star BASIC

program can be run under CP/M.]

[When copying files from CP/M to North Star, there are two options.

First is file-to-file copy without conversion.  Second, convert a CP/M

hex file to North Star type 2 executable file.  The second conversion

takes into account that ASM leaves gaps in the hex file where undefined

code was reserved.  INTERDOS fills these gaps with zeroes.  It assumes

that the file has the type and go address set properly so it ignores the

hex file addresses except for filling gaps.  In both options, the file

must have already been created under North Star DOS.]

[INTERDOS will work with single or double density disks.  The source

code is given because you may want to change the density or add additional

conversion formats of your own.  There are two equates at the beginning

of the program that you may want to change.  The first is NSBOOT and is

the address of the ROM on the North Star disk controller board.  This is

used to jump to in order to boot in the DOS.  In older systems, this

address was E900, but in the newer and double density systems, this

address is E800.  The second equate that may be changed is DENSITY.  This

is the density of the system and should be set to "S" for single density

or "D" for double or quad density.]

[Operation of INTERDOS is simple.  Mount a disk into the North Star

drive that contains the North Star DOS.  Then type the CP/M command

INTERDOS.  INTERDOS will load, print the signon message and ask

"REBOOT NORTH STAR DOS (Y/N)?".  At this point enter Y and the program

will jump to E800 (or E900) to boot DOS.  DOS will gain control and type

the * or +.  Type JP 100 to DOS to return to INTERDOS.  INTERDOS will

reexecute and again ask "REBOOT NORTH STAR DOS(Y/N)?".  This time type N.

Then it will ask "DESTINATION SYSTEM (C/N)? ".  Enter C to copy from

North Star to CP/M or enter N to copy from CP/M to North Star.  A

verification message will then be displayed.  Next it will list the

possible formats and ask for character 0 thru 5.  If you enter an

invalid format or a format that is invalid for that copy type (such as

North Star BASIC to CP/M BASIC when you are copying CP/M to NS), the

options will be relisted and you will again be asked to enter the format.]

[The program needs to know when to stop the copying process.  In most

North Star formats, the end-of-file mark is an 01.  It can also count

blocks and stop when the block count exceeds the amount listed in the

directory.  The program will ask "EOF USING 01 OR BLOCK COUNT (1/B)?".

Enter 1 if it should stop when it encounters an 01 byte or enter B if it

should stop when it reaches the block count.]

[Then the program will tell you to mount the disks and enter return.

This makes sure the disks are ready so that when it asks for the file name

it can search the disk to be sure the file is there if it should be.]

[Finally it will ask for the file names.  The CP/M file name is in the

standard format X:xxxxxx.ttt   If a carriage return is entered without

entering any characters, INTERDOS returns to CP/M.  The North Star

file name follows the North #Star conventions including the optional comma

and drive number at the end.  The file copy process will begin as soon

as the files are verified.  As the files are copied under all formats,

all printable characters are displayed at the console.]