TITLE:  SETOK

BY   :  IRV HOFF

DATE :  10 OCT 82

TOPIC:  PRESETTING OKIDATA PRINTER OPTIONS

This simple program allows OKIDATA users to preset a number of

options.  The printer can then be satisfactorily used with the TYPE

command (via CTL-P) or other normal listing programs.

These options include:  10 or 16.5 chars. per inch

64 or 80 chars. per line

6 or 8 vertical lines per inch

wide print

reset to normal default values

To use, turn on the Okidata printer and then call up SETOK.COM.

It will display various options on its menu.  Several selections can

be made at the same time or a number reentered if mis-typed.  If RET

is immediately typed it defaults to a standard configuration.

Default is:  10 characters per inch

80 characters per line

6 vertical lines per inch

After the appropriate answers have been typed, hit RET to leave

the program (or CTL-C).  The printer now has the options you selected.

The TYPE command (via CTL-P) may then be used or any other LIST routine

desired.

Obviously some options were not included.  The SETOK.ASM file can

be edited and then reassembled.  (ASM.COM works fine for this purpose.)

Even if the operator is not too familiar with using source code, there

are enough examples already included to make the job of adding some ad-

ditional feature fairly simple.

Irv Hoff

Los Altos Hill, CA  94022