MIDI DATA FILER ver 3.1, April 1993 by Kai Risku

================================================

This is a very easy-to-use program that will send and receive midi sysex

dumps on a PC. The dumps are stored as raw files and are compatible with

sysex dumps used by most sequencing programs. The dumps can be several

hundred kilobytes in size (limited by available memory).

The program requires a MPU-compatible midi interface and can be

configured for any base address (interrupts are not used at all).

How to save MIDI-data to a file:

================================

You just start MDF with a non-existent filename as argument. When MDF

reports 'Start dump' you can start a sysex-dump manually from your

synthesizer or press any key to abort. A dot will be displayed for every

sysex block received (so you can see that the computer is actually

getting some data).

After the dump is done, MDF will report how many bytes have been

received, save those in the file you specified and exit automatically.

The amount of data you can receive is limited by the available memory

below 640k (minus about 40k for MDF itself).

How to send the data back to the synthesizer:

=============================================

Oh, you have already guessed that one, but I just want to say it anyway.

Run MDF with the name of the file you want to send to your synthesizer

as the argument. MDF will send the contents of the file and inform you

of how many bytes has been sent. You can press ESC if you wish to abort

the transfer.

Configuring MDF using the MDF.INI file:

=======================================

When starting up MDF, the program will look for a file named MDF.INI in

the same directory as MDF.EXE is in. If the INI-file cannot be found,

one will be created using some default values. The file is in plain

ASCII format and can be edited with any ordinary ascii editor. These are

the valid options and what they specify:

Delay1=n        where n is a decimal integer specifying the delay

between each byte sent. A larger value means

slower sending.

Delay2=n        where n is a decimal integer specifying a delay

in milliseconds between each sysex block sent.

Timeout=n       where n is a decimal integer in milliseconds specifying

how long MDF should wait for more data before finishing.

Extension=ext   where ext is a filename extension with a maximum

of three characters. This extension will be used if

MDF is invoked with a filename with no extension.

If this option is missing, the default extension

will be SYX. Using filenames with no extension at

all requires either a period after the filename

on the command line, or removing the characters

to the right of the equal sign in MDF.INI.

Port=addr       where addr is the address of the midi interface. The

address can be given in decimal or hex. To specify

hex mode, you must either have '0x' before the address,

or the character 'h' after. The default address can be

written these ways; Port=816, Port=0x330 or Port=330h

If it does not work:

====================

- If your MPU-compatible interface is not installed on the default

port address 330h, you must set the correct address in MDF.INI.

- some synthesizers can choke if the computer dumps data too fast.

Increase the delays specified in MDF.INI to slow down the sending.

- There can be some problems when receiving a dump if the

synthesizer dumps data too fast and you have a slow computer.

You should therefore keep an eye on the size of the received

data as this will be too small if data has been lost.

- You cannot use handshake-dumps with this program, because

practically every synth has its own format for this.

Make sure that you start a one-way dump on your synthesizer.

Revision history

================

New in version 3.1:

Fixed bug that occasionally saved some garbage at the

end of the file when receiving. Tweaked the documentation.

New in version 3.0:

Sysex dumps are no longer limited to 64 kbytes. Receiving

sysex data is only limited by your dos memory (below 640k).

The receive timeout can now be specified in MDF.INI.

Program selfcheck added and large portions of the code

rewritten to improve the program in several ways.

New in version 2.6:

The address of the midi interface can be set in MDF.INI

New in version 2.5:

You can now change the default filename extension

using MDF.INI

New in version 2.4:

Ctrl-C is now trapped, so you cannot break out of the

program and leave the midi interface in UART mode.

New in version 2.3:

Added an INI-file for user-configurable delays while

sending. Modified text output and removed some

unnecessary texts.

New in version 2.2:

More code cleanup. Incoming data during sysex-send

to synthesizer will be sent back out when possible.

Error-handling improved.

New in version 2.1:

The default extension MDF for files created was changed

to SYX for compatibility with other software.

Active sensing has earlier caused some trouble, but

this has now been fixed as well as a small bug causing

a time-out if you don't start the dump fast enough

after you started the program.

New in version 2.0:

Document added (this piece of text). Improved routines

with faster code. Corrected some minor bugs. This was

the first "official" release.

#include <std_disclaimer.h>:

============================

Copyright (c) 1990-1993 Kai Risku

MDF is offered to you "AS IS" without any warranty. This software has

been thoroughly tested but no guarantee is given that it will work on

every computer. The copyright owner may not be held liable for any

damages, direct or consequential, which may result from the the use of

this program.

This program is freely distributable. You may use the software and

share it with all your friends (and foes) as long as the program is

supplied in its original, unmodified form, which includes this

documentation. This program must not be distributed for profit.

If you have any questions, ideas, suggestions for improvements or if

you are interested in the source then feel free to contact me...

A nice postcard would be appreciated!

Snailmail:      Kai Risku         Email (internet):  Kai.Risku@hut.fi

Kronvik

65410 Sundom

Finland