DISKSECURE II version 2.42

Hard Disk Protection Program	      Copyright (c)1990, 1991, 1993, 1994

by Padgett Peterson

All Rights Reserved

I  - Why low-level protection ?

II  - What is DISKSECURE II ?

III  - How is DISKSECURE II Loaded ?

IV  - How is DISKSECURE II Removed ?

V  - Malicious Software (viruses) and DISKSECURE II

VI  - Return Codes

VII  - Licensing - Now FreeWare

VIII  - Weasel-words

Appendix A  - Hidden Sectors/Changing MBR/Changing DBR

Appendix B  - DS2BYP and DS2MOVE

Appendix C  - Other Files

I - Why Low-Level Protection ?

Today there are many anti-viral software products available

for the IBM-PC (and clones) platform running MS-DOS, OS/2, and UNIX but

nearly all are invoked following load of the operation system (usually

from CONFIG.SYS or AUTOEXEC.BAT in MS-DOS).  In many cases this is too late

for effective protection from common boot sector infectors (BSI) such as

STONED, MICHELANGELO, or the MUSICBUG.  In the case of the "stealth"

variants, even state-of-the-art anti-virus routines such as the McAfee

programs must rely on detection of a viral signature in memory rather than

detection of infected files.

At the operating system level, too many extra programs have been

piled upon the basic disk access routines to be able to reliably separate

the actions of legitimate drivers and TSRs from the activity of malicious

software.

There is a point at which executable code may be placed that

will always be executed during the boot of a hard disk when the low

level activities have not yet been masked - during the execution of

the Master Boot Record.  At this point the organization of a PC is defined

according to the IBM BIOS specification. In most cases this structure is

impossible for malicious software to subvert without causing a detectable

change.  As of August, 1993 there are no known boot sector viruses that

can bypass detection at this level.

Additionally, there are many access control packages available

that provide strict control and full encryption of systems.  DISKSECURE II

does not attempt to do this, rather it is intended as a simple, unobtrusive

primary access and virus control mechanism that does not require

administration, and which may be installed/removed/modified with a simple

menu-driven interface.

II - What DISKSECURE II Does

Simply put, DISKSECURE II replaces the Master Boot Record on

a hard disk with its own code.  The necessary elements of the original

partition table are stored on an unused part of the disk and presented

to the authenticated operating system as required. Unauthorized boot

programs (such as the BRAIN, STONED, or AZUSA viruses) will not be

able to access the partition table as expected.

DISKSECURE II has multiple elements: three are contained in the file

DISKSEC2.EXE supplied with this document.  The first is an interactive

installation/maintenance/removal module that allows the user to activate,

change, or remove DISKSECURE II from a hard disk. The second is a replacement

for the hard disk partition table that performs load-time checking, prevents

DOS access to the hard disk if booted from a floppy, and contains the

password access routine if implemented.  The third element is a resident

protection program that takes up 1k of RAM memory and protects the partition

table, hidden sectors, and boot sector of a conventionally partitioned hard

disk.

To this, DISKSECURE II adds Novell Server compatability, Windows

3.1 32BitDiskAccess compatability, and the ability to transform itself

from a 1K TSR at the Top of Memory into a 304 byte TSR in low memory (DOS

3.2 and above only).

Once loaded, DISKSECURE II will prevent access to the fixed disk from

DOS if the resident portion of DISKSECURE II is not in memory.  If password

access has been invoked, this password must be given before DISKSECURE II

will become resident, allow the OS to load, and before DISKSECURE II can be

changed or removed.

In the event of accidental booting from an infected floppy disk,

DISKSECURE II adds the ability to automatically detect and disinfect any

virus that does not overwrite the MBR. Such viruses are rare though these

will render the system unbootable. In this event, the Recovery Disk has two

different means for recovery (use of the DiskSec2.exe and the Dspart.dat

files).

Recently, a single easily detectable virus has been seen which

blindly corrupts the Dos Boot Record. DS II will detect these and removal

is generally a matter of booting through DS II to a floppy and using the

BYPASS module to SYS the fixed disk. This mechanism should work so long as

the fixed disk is accessible on a floppy boot (cntrl key depressed).

However, the user must realize that should a boot from an infected

floppy occur before DS II loads, anything could happen and the best that

can be done is to detect that action. See the comments on BIOSes which allow

boot disk selection.

DISKSECURE II adds the capability to boot from a conventional floppy

if the Ctrl key is held down during boot. No special Maintenance floppy is

needed. This should remove all opposition to setting those BIOSes which

permit it to boot from the fixed disk only.

Other elements of DISKSECURE II include an automated installation

program  (DS2INST.BAT) - installation may be dome manually if desired,

a program to verify proper operation of DISKSECURE II (DS2CHK.EXE), a

program to move the DISKSECURE II TSR into low memory (DSMOVE.SYS), and

a program to allow compatability with incompatible programs (DS2BYP.EXE).

III - How  DISKSECURE II is Loaded

The modification program may be loaded either from the fixed

disk or from a separate, bootable floppy (recommended).  It is designed

to recognize if DISKSECURE II is resident, will make many checks to

ensure the integrity of the program, and will warn the user of anything

unusual that it finds.

If DOS 6.0 or above is in use, it is suggested that DS II be

installed following a "Bypass Boot" (press F5 during boot so that

CONFIG.SYS and AUTOEXEC.BAT are bypassed)

THE DISKSEC program is invoked by executing from either the DOS

prompt (automatic installation from other OSs is not yet available) or a

batch (DSINSTALL.BAT) file.  DISKSECURE II will check the number of fixed

disks present and verify this with the user before proceeding.  Following

this, DISKSECURE II will require the user to input a password if this

additional protection is in use before proceeding.  At this point, a

first-time user will be presented with a warning message before the menu

is presented.  The user may then select a choice from the menu (the menu

will change to accommodate the state of DISKSECURE II in the system).

During installation, the user will be asked to save the partition

tables to a file (default: DSPART.COM).  This file should be copied to

a known clean, bootable floppy disk.  If all else fails, the system may be

booted from the floppy, and DSPART executed. Even if the disk is

unrecognizable by DOS, this will replace the partition table.  WARNING: if

this file is executed on a machine other than the original or the partition

table has been changed by repartitioning all data on the disk could be lost.

Where multiple machines are to be protected, it is suggested that

each machine have a unique and identifiable recovery disk and DSPART.DAT

be renamed to identify it with each creating PC. However recovery files

for multiple machines can be stored on a single floppy along with a single

copy of DiskSec2.exe.

Should it be necessary to repartition a disk or the operating

system upgraded, it is necessary that the simple removal procedure be

followed first as DISKSECURE II will trap any attempt to write to the MBR,

hidden sectors, or the boot record. DS2BYP should not be used for this

function as essential signatures will have changed and on the next boot

the machine may be restored to the previous condition.

Also provided is the program DS2CHK.EXE.  This will allow the user

to check if DISKSECURE II is in memory - on activation, nothing will return

if DISKSECURE II is not active - the DISKSECURE II logo will display if it is

active.  An errorlevel of (0) will return if DISKSECURE II is not found,

(1) if DISKSECURE II is resident but not in control (e.g. QEMM 6.0+ "stealth"

in use), and (2) if in memory and in control. DS2CHK.EXE may be invoked

as part of a network login script to verify proper use on a client PC.

The other two major programs included are DS2MOVE.SYS (explained

above) and DS2BYP.EXE. The second should be used with care and only when

necessary to accommodate programs requiring low level access. See also

Appendix B

Note: After activation, if booted from a floppy DOS will not recognize

the hard disk(s) and, if additional removable media are present,

DOS may recognize them as the C: drive.  This will not affect

the operation of DS2CHK.EXE nor will it endanger the data on the

fixed disk.  If DISKSECURE II is removed, on the next boot the fixed

disk(s) will again be recognizable by DOS from a floppy. The disk will

always be recognized when booted from a floppy disk through DISKSECURE

II (by holding the Ctrl key down during boot) so long as DISKSECURE II

maintains validity. (if failure occurs, see RECOVERY above).

Use of the DSPART.DAT file as outlined above from a standard DOS

bootable floppy will recover the disk even if DOS reports the

disk unavailable. On the next boot, the drive will be again

accessible, however DISKSECURE II will have been removed and must

be re-installed to restore protection.

IV - How DISKSECURE II is Removed

In the event that it is necessary to remove DISKSECURE II from a PC,

the best way is to utilize the DiskSec.EXE program either from the

MAINTENANCE floppy, or from the c:\ds2 directory. When DS is resident, the

program will detect this and modify the menu to include a REMOVE option.

If a password is in use, it will be necessary to provide this also.

Secondly, the machine can be booted from a regular floppy disk

and the DiskSec2.exe program executed. If a password is in effect this

must be entered for activity.

If the disk should become corrupt, often booting from floppy and

running the DiskSecure2 program will enable recovery. Should this fail,

the DSPART.COM program on the RECOVERY disk may be used for an emergency

recovery of the low levels after which the machine may be rebooted and

conventional recovery tools used.

V - Malicious Software and DISKSECURE II

DISKSECURE II is a software program.  Consequently, even though it

is the first software loaded from a fixed disk it CAN be infected or

damaged , should an indadvertant boot from an infected floppy disk take

place, even though DOS will not recognize the fixed disk .  Only hardware

in the form of a ROM extension or custom BIOS can prevent this.

One effective means with newer BIOSes is to set the CMOS so that

the PC will always boot from the C: drive first. Since DISKSECURE II

allows transfer of a boot to floppy by holding the Ctrl key down during boot,

selection of the A: drive need never be automatic. This combined with

DiskSecure II is effective against every known virus or viral technique

(and a few that haven't been seen yet - don't ask 8*).

What DISKSECURE II can and does do is to recognize immediately

when it is loaded that an infection has occurred, display an error

message as listed below, and refuse to proceed with the boot process until

corrected (in most case this will be automatic and wait only permission from

the user. In rare cases, booting from a floppy will be necessary for

correction however the machine will not boot unless DISKSECURE II's

multiple redundant criteria are met.

Additionally, certain destructive viruses (such as AZUSA) can

destroy DISKSECURE II. In this event, the disk will be unable to boot and

the drive will be unrecognizable from a floppy. The use of DSPART.DAT,

stored on a bootable floppy, renamed DSPART.COM, and executed will

restore the partition table to a usable condition. In any event, the

attack will be immediately noticeable and containable.

DISKSECURE II boot error messages: see ERROR.MSG file.

Additionally, some malicious software (as well as some low-level

programs) may try to write to DISKSECURE II protected areas on the disk.  In

this case a message may appear on the screen "DISKSECURE II TRAP: x" where x

indicates the trapped function.  If this should happen during a legitimate

program, use of DS2BYP will be necessary. This should be done only under

known safe conditions.

A DISKSECURE II trap will occur on an attempt to write to any of the

"hidden" sectors as well as the first partition's boot sector, or an

attempt to format any sector on the protected disk(s) (note: many OS format

programs do not actually "format" the disk, only the low-level formatting

such as done by programs that set the interleave of a disk may do this.  (A

MS-DOS 3.3 "FORMAT" will not trigger a DISKSECURE II trap until it is nearly

complete and information may be lost).

Generally, this will only occur if a program attempts to change disk

interleave, boot record/partitioning, or on complete replacement of the

operating system (Central Point Software's COMPRESS and Peter Norton's

SpeedDisk are not affected, Peter Norton's CONFIGUR, MS-DOS FDISK, and

Steve Gibson's SPINRITE are).

Further, DISKSECURE II has proven safe with all major disk

compression programs including Stacker, SuperStor Pro, and DblSpace as

well as nearly all other anti-virus programs though some may flag

its activity.

DISKSECURE II cannot protect the FATs or Directory structure since

the user must be able to modify these.  Other programs such as Enigma-

Logic's VIRUS-SAFE, Symantec's NAV, Fischer's PC-WATCHDOG, the McAfee

Programs, the Dr.  Panda Utilities, Ross Greenberg's FLUSHOT, and Fridrik

Skulasen's F-PROT exist for this function.  DISKSECURE II was created

to plug a hole that exists under the operating system so that a reasonable

expectation of a clean system can be had when the "C:\>" prompt appears.

VI - Return Codes

On termination (if not a reboot) DISKSECURE II returns various error

codes to indicate the reason.  These may be used by a batch file

for checking/corrective action, however DISKSECURE II is primarily

a manual, interactive program.

DISKSEC2.EXE

user terminated codes

code (hex)

0   (0)	Program completed with no errors

1   (1)	Following warning message

2   (2)	Following TOM/COMPARE message - possible virus

3   (3)	Following number of fixed disks message

7   (7)        Following save to file request

program terminated codes

code (hex)

8   (8)	No hard disk responding

9   (9)	Disk access failure

10   (A)	No active partition table located

11   (B)	Error in sector one on disk - possible virus

12   (C)	Active partition table not in proper place - possible virus

14   (E)        No "Hidden Sectors" on disk (used early version of FDISK).

or Changing MBR (see actual error message) See Appendix B

DS2CHK.EXE

0    (0)	DISKSECURE II not present in memory

1    (1)        DISKSECURE II present in memory but not in control

(low interrupt vector seen only with QEMM 6.0+ "stealth")

2    (2)        DISKSECURE II present in memory and in control

4    (4)	DISKSECURE II has been disabled in some other manner

8    (8)        Disk did not respond (should not occur)

VII - Licensing

DISKSECURE II is copyrighted material distributed both as individual

copies and on site/entity licenses.

With version 2.4, DISKSECURE follows the trend I began with the

FixUtilities of being FreeWare to individuals, however retaining Copyright.

It is not public domain material. It may not be altered in any way or

distributed except as a complete package (see the DSxx.VAL file for a list

of contents).

Customized versions including special switches, logos, encryption,

full partition protection, enforced acceptance, and disclaimers such as:

"Property of the XYZ Corporation & is for Authorized Users Only.

The right is reserved to monitor any and all transmissions, keystrokes,

and storage on this system."

are available on a site licensing basis.

Donations cheerfully accepted 8*) Padgett Peterson

POB 1203

Windermere, Florida, 32819

407.352.6027

VII - Weasel-Words

No guarentee for usefulness or fitness of any kind is made and the user

is responsible for whatever happens. While I have made every reasonable

effort to make sure that this software will do what it is supposed to,

I cannot check every possible combination of equipment/software possible

(though heaven knows I've tried). I will try to fix any reported and

verified bugs.

Appendix A - Hidden Sectors / Mutable MBR

While all disk partitioning schemes since the release of PC-DOS

3.0 in early 1984 have aligned partitions on cylinder boundaries,  very

early disk partitioning programs (e.g.  FDISK 1.00) did not perform this

alignment.  DISKSECURE II relies on this alignment for installation and

protection, therefore the installation procedure verifies that no partition

violates this requirement.

Should a disk be found on the system (DISKSECURE II checks all fixed disks),

the installation process will terminate with the warning "No Hidden

Sectors...".  In this case, it will be necessary to repartition the disk

using a later version of the partitioning software.

Note: It is possible to be running a later version of DOS or other OS

with a disk partitioned using the early scheme so the DOS "VER"

command cannot be used to reliably test for this condition.  Peter

Norton's DI (DiskInfo) and other software will report the number

of "hidden sectors" on a disk.  This value should be equal to the

number of sectors per track.

Changing MBR

Another condition that DISKSECURE II cannot cope with is that

a very small number of disk controllers write directly to the MBR at

intervals. This has only been observed on early XT hard disk units,

however the integrity of a DISKSECURE IId disk's MBR must be maintained.

If discovered at load time, the installation process will issue a warning

and the installation will abort. If this should occur during a boot,

the  MATCH ERROR termination will occur and the recovery disk should

be used to restore the disk. Re-installation should not occur until the

cause (viral or otherwise) has been determined.

Changing DBR

Even rarer are the operating systems which write often to the

DBR but they do exist. Zenith ZDS 3.0, 3.1, and 3.2 plus a few early

HP Vectras are known to exhibit this behavior. Should periodic "DS II

TRAP: 3" messages occur during the boot process, this may be the cause

since DS II will protect the DBR from writing by any software. Certain

later OS software may also exhibit this charactorisitic, for example MS

-DOS 5.0 & later will attempt to change the Disk serial number in the

DBR if a LABEL command is issued & two or more TRAP errors will display.

The label will however be properly changed.

One virus has been seen which on boot from floppy will blindly

write to a common DBR location. If DS II is not running then there is

no means of protection from this. It will be detected on the next full

boot. (see section 2). It is unlikely that this virus will be very

successful.

Appendix B - DS2BYP and DS2MOVE

The two special adjunct files provided with DISKSECURE II have

unique properties that are only functional with versions of DOS of 3.2

or above.

DS2MOVE.SYS is the most important as it allows moving of DISKSECURE II

from a 1k area at the Top of Memory (a 640k machine will report 639k)

to a 304 byte area of low memory restoring the full 640k to DOS. This

program is placed in CONFIG.SYS (best if first driver loaded except when

QEMM DOSDATA driver is in use) and has no switches.

DS2BYP.EXE is for use *only* when a program proves completely incompatable

with DISKSECURE II (such as WIndows 3.1 in 32BitDiskAccess mode). It

is invoked as follows DS2BYP <drive>:<path><filename.ext>. DS2BYP

only accepts programs with .COM and .EXE extensions and the drive, path

and filemname must be fully expanded (no PATH searches or wildcards

are permitted). Only one program will run on each invocation. This

rigorous approach is to prevent as far as possible any possibility of a

spoof or companion attack. Use with caution.

Appendix C - Other executable and batch files

DS2INST.BAT  - a batch file to be used when installing DISKSECURE II

from a floppy disk. It will create a directory \DS

on the C: drive, copy the files from the floppy

into that directory, patch the AUTOEXEC.BAT file

to include  verification of DISKSECURE II in memory on

boot (recommended but may be omitted), and invoke

the main program for installation.

DS.B         - Batch file commands to be added to AUTOEXEC.BAT for

checking referred to in DSINSTAL.BAT

ASK.COM      - 10 byte file for making .BATch files interactive. ASK

will wait for a keystroke and return an errorlevel

that may be used by an "IF ERRORLEVEL" construct.

CHK512.COM   - Short .COM file to identify certain incompatable drives

CHKINT13.COM - Short .COM file to verify a "clean" path to the BIOS disk

access.

DOS32.COM    - Short .COM file to verify use of DOS 3.2 or later.

QEMMST.COM   - Short .COM file to detect use of QEMM 6.0 & above "stealth"